Kinetic Sliding Door System

Kinetic ALIGN Sliding Door

18 May 2013

Installer’s Guide

General Introduction

The purpose of this Installer’s Guide is to illustrate the specific sequence and method for construction of the Optima Kinetic ALIGN Sliding Door. It is assumed that the fitting teams carrying out the installation have the necessary skills to set out, operate the tools required and install the system to the required standard.

It is further assumed that good practice for installing factory-finished linear track sections will be followed at all times. This will include:

- Avoid cut lines/butt-joints coinciding on multi-section tracks – example: Floor track and glazing beads

- Ensure that where butt-jointing aluminium sections, the painted end of the extrusion is cut off first. This will ensure a good joint by removing the uneven end that can occur during the extrusion and painting process. When referenced in the following text, a full length is assumed to have been trimmed.

- Pre-drill and countersink all fixing holes to ensure that all countersunk fixing screws sit flush.

- Fix tracks to all abutments with fixings at not less than 300mm centres.

- To maintain the acoustic integrity ensure all abutment tracks are properly sealed to the structure with either the factory supplied (fitted) foam tapes or proprietary acoustic sealant. There should be no visible air gaps between track and structure.

It is the installer’s responsibility to select fixings appropriate to the fixing substrate, since it is not possible to anticipate the nature of projects in advance. No recommendations will be made in this document. When considering the choice of fixings it is essential that the selection is capable of supporting the designed line load of 0.36kN/m

The text of this Installer’s Guide, as far as possible, has been written in an abbreviated form in order to keep the document as short as possible. Where a particular component or track section is mentioned without an accompanying sketch, it will be annotated in bold text. The reader should refer to the appendix at the back of the document to cross reference the component code with an illustration for identification.

Typical Arrangement

Door omitted for clarity

Installation Sequence

The following is the installation sequence based on 12/12.8mm thick fixed glazing as recommended by Optima Products Limited. Deviations from this sequence are permitted provided that the quality of the completed installation is not compromised.

1	Prepare the Surrounding Structure

- Ensure all abutments are complete and preferably painted.

- The ceiling must be capable of supporting the weight of the head track and receiving its fixings. With a plasterboard ceiling, where fixings into the ceiling framework are assured, this may be sufficient. However, it is preferable and with tiled ceilings, essential that a suitable patress is positioned above the ceiling on the line of the head track. This will be ideally 18 or 25mm plywood. It may also be necessary to brace the patress back to the slab to eliminate any lateral movement in the event of a line load being applied.

2	Install the Head Track

- The Kinetic head track (0209) should be cut for a continuous tight fit between vertical abutments with 90° corners cut with two clean 45° mitres.

- Drill the track alternately through the door and glazing portions to ensure a stable fixing.

- Where a 3-way junction to a glazed partition is required (example: glazed fin to cross-wall), the centreline of the cross-wall/fin must coincide with a joint in the Kinetic head track and the head track must be notched to allow the fin glass to pass through.

- For a 3-way junction with single glazed Optima 117 plus cut a 13mm wide notch from the back face of the track to the inner face of the middle wall (39.25mm). This will allow the Optima 117 plus head track to locate.

- Cut a further 9mm wide notch (4mm shoulder) to a depth of 7mm. This will fully break through to the glazing channel allowing the cross-wall/fin glass to pass through.

Note – This detail cannot be applied where a deflection head is to be used

- For a 3-way junction with centrally single glazed Revolution 54 cut a 27.5mm wide notch from the back face of the track to the inner face of the middle wall (39.25mm). This will allow the Revolution 54 head track to locate.

- Cut a further 9mm wide notch (18.5mm shoulder) to a depth of 7mm. This will fully break through to the glazing channel allowing the cross-wall/fin glass to pass through.

- For a 3-way junction with double glazed Revolution 54 cut a 27.5mm wide notch from the back face of the track to the inner face of the middle wall (39.25mm). This will allow the Revolution 54 head track to locate.

- Cut a further 17mm wide notch (6.5mm shoulder) to a depth of 7mm. This will fully break through to the glazing channel allowing the cross-wall/fin glass to pass through.

- For the tracking details of the Optima 117 and Revolution 54 fins, refer to the respective Installer’s Guides

- Before fixing head track in position, ensure that the two grooves on the top surface are fitted with acoustic foam tape and insert the door carriages and buffer stops in the correct order: hold clip – trolley – trolley – hold clip.

The trolleys should have the banjo hangers screwed in before inserting into the track

Ensure that the carriages are inserted the right way up. The ribbed edge of the trolley should face down.

3	Install the Vertical Abutment Tracks

- For vertical abutments cut a 25mm glazing channel (0322) for a tight fit between the Kinetic track and the floor. Insert glazing gasket (2122) into one edge before installing the tracks.

- Where a single glass module may be difficult to glaze, a two part glazing channel (0325) with removable bead (0328) can also be used. The removable bead should be on the outside of the room. These also use slide-in gasket (2122), inserted before fixing track and push-fit gasket (2121) at the time of glazing.

4	Set out the Door Opening & Size the Door Leaf

- Mark the overall door frame width on the floor. This will be 1196mm for the standard door width (ref: Optima Designer’s Guide). This will define the extent of the glazing floor track.

- Order the door leaf using one of these Optima order templates:

  - 300065	LH Door Leaf with Lock

  - 300066	LH Door Leaf without Lock

  - 300067	RH Door Leaf with Lock

  - 300068	RH Door Leaf without Lock

Note: The door leaf will have 6mm clearance to the Kinetic track and 14mm clearance to the floor. This allows for a floor finish (carpet) of 8mm.

5	Install the Glazing Floor Track

- For glazing floor tracks cut two part glazing channel (0325) and removable bead (0328) for a tight fit between the vertical wall abutment tracks and the door frame set-out lines. The removable bead should be on the outside of the room. 90° corners should be cut as clean 45° mitres.

6	Measure and Install the Fixed Glazing

- Glass panels should be measured as follows:

- Glass panel height: Raised floor to u/s ceiling minus 15mm

- Penetration into vertical abutments: + 15mm

- Penetration into door reveal channels: + 30mm

- The Kinetic system is designed so that the fixed glazing supports the weight of the door. The calculation above allows for a nominal 4mm rigid glazing packer above the glass and 7mm between the raised floor and underside of glass. This will ensure that the glass is installed rigidly into the tracks. The packing dimensions can be altered to suit any glass dimensional tolerances.

- For locking doors the adjacent fixed glass panel is cut with a notch to receive the lock case. Consequently, the glass panel must be toughened glass. Non-toughened laminated glass may not be used.

- The notch must be set to the correct level to ensure that the lock bolt locates correctly. The dimensions for measuring the glass height are the same as quoted above but are illustrated here for clarity

7	Install the Door Frame (Door Reveal Channels)

- The door reveal channels (0213) should be cut 1mm shorter than the distance from Kinetic track to floor.

- Insert slide-in seal (2364) into the face of the reveal channels so that the blades both point towards the closing direction of the door. In addition, insert the slide-in glazing gasket (2122) before fixing the channel in place

- With a fixing bracket (0024) inserted into each end of both sections so that the narrow leg with the fixing holes point towards the door opening, position the sections over their respective fixed glazing panel and fix plumb to the floor and the inside soffit of the Kinetic track using the wafer head screws (0014) provided.

8	Complete the Fixed Glazing

- Seal the glass into the glazing tracks by inserting the push-fit glazing gasket (2121)

9	Fit the Floor Guide

- Check the distance between the two door reveal channels. This will be 1115 or 1116mm for a standard door (see detail). The floor guide length is calculated as: 2W-125mm.

Example: (2 x 1115) – 125 = 2105

- Cut two lengths of floor guide (0210) and fix back to back at 300mm (max) centres, with the centreline at 20mm from the face of the partition cap. Ensure screws are countersunk.

10	Hang the Door Leaf

- Rotate the banjo hangers until the centre of the face plate is 31mm below the underside of the track.

- Place a 14mm rigid block (preferably timber) on the floor either side of the floor guide and 150mm inboard from each side of the opening as a rest for the door.

- Stand the door upright on the rest blocks with the base positioned on the door centreline and with the two guide rollers (at each end of the door) sitting in the floor guide, aligning the banjo hangers with the holes in the top edge of the door.

- Remove the banjo face plate and insert into the door. Replace the plate and loosely tighten the screws so that the cap is not loose.

- Remove the rest blocks and allow the door to hang before fully tightening the banjo face screws.

11	Finish the Door Leaf

- Peel the skin from the tape on the top rail sections (0346 and 0347) and bond to either face of the glass leaf, aligned with the top edges of the door stiles.

- For locking doors, assemble the lock housing:

- Assemble the lock housing by clamping the Main Body  (inside face of glass) to the Back Plate (outside face of glass) using the M5 x 35mm machine screws provided, ensuring the Neoprene Gaskets are positioned against the glass.

- When clamped in position the housing should be touching the edge of the door reveal channel

- Position the Hook Lock case in the Main Body and close the housing with the Front Plate, securing with the two M4 x 20mm machine screws provided.

- Insert the cylinder and secure with the M4 x 45mm machine screw provided.

- Ft the door handle.

- Set the position of the in-rail hold clips by sliding and securing with the Allen key provided:

- Snap the hold clip on to the nose of the trolley and slide the door to its required extreme (open/closed) positions and secure by tightening the grub screw at the outer end of the unit. The inner grub screw is used to adjust the tightness of the grip.

- In the fully open position the hold clip should be positioned such that the door is projecting 247mm from the edge of the fixed glazing.

12	Finish the Head Track

- To close off the redundant door track section beyond the door’s maximum opening travel, cut lengths of partition head infill (0214) for a tight fit and snap-fit in place.

- To close off the glazing track section across the door opening, cut door head seal carrier (0341) for a tight fit.

- Insert a similar length of slide-in seal (2364) with the blade pointing upwards and snap-fit in place with the seal in contact with the face of the door top rail.

Appendix A	Installing with a Deflection Head

The installation of the system with a deflection head follows a similar method to the non-deflection version described above. However, there are deflection-specific details and additional deflection head components to install first.

- The Kinetic ±25mm deflection head track (0215) should be cut for a continuous tight fit between vertical abutments with 90° corners cut with two clean 45° mitres.

- Before fixing the deflection head track in position, ensure that the two grooves on the top surface are fitted with acoustic foam tape

  - Where a 3-way junction to a glazed partition is required (example: glazed fin to cross-wall), the head track must be notched to allow the fin glass and head track to pass through.

- For both single and double glazed fins (Revolution 54), cut a 55mm wide x 65mm high slot, centred on the fin centreline. The Revolution 54 deflection head will butt directly against this opening.

- Before installing the Kinetic track, cut matching lengths of ±25mm inner channel (0340) and fix to the top surface of the Kinetic track ‘open side up’ with both edges aligned using countersunk speed-tip drywall screws driven up through the Kinetic track.

- Insert the assembled track into the deflection head track so that there is 25mm projecting and hold in place using a series of temporary low tack tape straps. The straps will be removed progressively as the glass is installed and is able to support the track.

Appendix B	Schedule of Components

|  | 0346 0347 |  |
| --- | --- | --- |
| 0209xxxxxxxx Head Track | 0346xxxxxxxx Top Rail (inner) 0347xxxxxxxx Top Rail (outer) | 0210xxxxxxxx Floor Guide |
| 3100mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit | 1 of each (top rail) x cut for door width Supplied as part of the Kinetic ALIGN Sliding Door kit | 2 x 3100mm lengths Supplied as part of the Kinetic ALIGN Sliding Door kit |
| 0211xxxxxxxx Partition Head Infill | 0213xxxxxxxx Partition End Cap (40mm) |  |
| 1 x 3100mm length Supplied as part of the Kinetic ALIGN Sliding Door kit | 1 x 3100mm length Supplied as part of the Kinetic ALIGN Sliding Door kit |  |
| 0215xxxxxxxx ±25mm Deflection Head | 0340xxxxxxxx Deflection Inner Channel | 0325xxxxxxxx Glazing Channel |
| 3100mm lengths Supplied to separate order. Not part of Kinetic ALIGN Sliding Door Kit | 3100mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit | 3100mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit |
| 0328xxxxxxxx Removable Bead | 0341xxxxxxxx Door Head Seal Carrier | 236441300031 Slide-in Seal |
| 3100mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit | 1 x 2000m length Supplied as part of the Kinetic ALIGN Sliding Door kit | 3 x 3000mm length Supplied as part of the Kinetic ALIGN Sliding Door kit |
| 212141300031 Push-in Gasket | 212241300031 Glazing Gasket (slide-fit) | 001900000057 Kinetic Accessory Pack |
| 3000mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit | 3000mm lengths Supplied to separate order Not part of Kinetic ALIGN Sliding Door Kit | 1 x Pack Supplied as part of the Kinetic ALIGN Sliding Door kit |
| XXXXxxxxxxxx Kinetic Fixing Clip Pack | Hook Lock Assembly |  |
| 1 x Pack Supplied as part of the Kinetic ALIGN Framed Door kit | 1 x unit Supplied as part of the lockable Kinetic ALIGN Sliding Door kit |  |

Appendix C	Amendment Record

| Amendment Date | Details | How Communicated | Authorized by |
| --- | --- | --- | --- |
| 8 July 2011 | First issue as part of the Formal Product Launch | Product Launch Documentation | P. Long |
| 31 August 2012 | Revised for re-modelled ALIGN door | Placed on Optima website | P. Long |
| 25 March 2013 | New Order Template references incorporated | Placed on Optima website | P. Long |
| 18 May 2013 | Revised to incorporate product upgrades | Placed on Optima website | P. Long |